Technical Report

NDOT Concrete Mix Design Automation

Group C: Paul Fallon | Levi Woockman | Arturo Garcia-Francisco

CIVE 202: Civil Engineering Analysis II | Dr. Kaycie Lane | Spring 2026

Client: Nebraska Department of Transportation (NDOT)

Submitted: February 24, 2026

- Introduction

  - Background

The Nebraska Department of Transportation (NDOT) currently calculates concrete mix designs for highway construction using an Excel workbook. The workbook requires users to manually input material values and design parameters into the correct cells within the Excel workbook to calculate how much of each material is required per cubic yard of concrete. There are downsides to the current workflow such as inputs needing to be re-entered manually, embedded formulas in spreadsheet cells being difficult to audit, and lack of connectivity between the Excel workbook and other digital tools.

To address the issues, NDOT has contracted group C to program the excel solver workbook into a Python program. The goal is to make the calculation process easier to repeat, maintain, and create transparency. The code was created in a Jupyter notebook which will guide the user through individual inputs that performs calculations and prints out a clean, easy to read report.

  - Objectives

The four main objectives for this project were:

- Translate Excel Formulas: Rewrite all 11 calculations from the NDOT Mix Design tab as Python functions.

- Sequential User Inputs: Prompt the user to enter each design mix value in the same order as the Excel worksheet.

- Professionally Formatted Output: Print a weight chart for one cubic yard of concrete in a clear, readable format.

- Verify Output: Research four realistic concrete mix design scenarios and run through the Python program. Compare the output of the Python program to the output for the same four mix designs using the original N

  - Resources

The following NDOT documents were used as the references for this project: the Concrete Mix Design Submittal Workbook (NDOT, 2024a), the Report on Optimized Aggregate Gradations for Portland Cement Concrete (NDOT, 2019), and the Standard Specifications for Highway Construction (NDOT, 2017).

- Methods

  - NDOT Concrete Mix Design Submittal Workbook Analysis

The first step of the project was to download the “NDOT Concrete Mix Design Submittal Workbook” from the NDOT website. This project is specifically focused on the “Mix Design” tab within the NDOT excel workbook. The required user inputs include weight of materials, specific gravities, water content, water-cement ratio, and aggregate proportions. The outputs from the “Mix Design” tab of the NDOT Excel workbook are the volume of all mix design components and final material weights. All values are based on producing one cubic yard of concrete.

Each cell within the NDOT “Mix Design” tab is labeled with a corresponding capital letter and all formulas within the tab reference those cell labels. In order to maintain traceability back to the original NDOT Excel workbook, the letters corresponding to each cell were including in the naming convention of the Python code variables.

All 11 formulas use the absolute volume method for concrete mix design. This method works by calculating the volume that each ingredient: cement, fly ash, water, and air, takes up in one cubic yard of concrete. Once those volumes are accounted for, the remaining space will be assigned to aggregate. The aggregate volumes will then be divided between fine, coarse, and other based on the proportions that are entered by the user.

  - Convert Excel Formulas to Python Functions

All formulas within the NDOT “Mix Design” tab of the Excel workbook were recorded and converted into Python functions. We used 3 key steps to complete these formula translations.

- Define All Constants: The Excel formulas use constants like 62.4 (unit weight of water in lb/ft³) and 27 (cubic feet in one cubic yard) written directly into cell formulas. In Python, these are defined once at the top of the notebook as unit_weight_water = 62.4 and cubic_yard_ft3 = 27, then referenced by name in every function. This makes the code easier to read and avoids errors from typing a number incorrectly in multiple places.

- Variable Naming Conventions: Each cell within the NDOT “Mix Design” tab of the Excel workbook has a corresponding capital letter. All variables in the Python Code referenced the corresponding letters to allow traceability back to the original Excel cell (e.g., weight_cement_A, sg_fly_ash_K).

- Dependency Order: Excel automatically recalculates formulas in the correct order when an input changes. Python runs code line by line, so the calculations had to be arranged in the right sequence manually. Water weight (Q) is calculated first because water volume (W) depends on it. Then all component volumes (R through W) are calculated, followed by total aggregate volume (X), and finally the aggregate weights (Y, Z, and AA).

Table 1 Excel to Python Translation

| Var. | Excel Formula | Python Function | Output |
| --- | --- | --- | --- |
| Q | (A+B+C+D)*E | calculate_water_weight_Q() | Water weight (lb/yd³) |
| R | A/(J*62.4) | calculate_volume_cement_R() | Cement volume (ft³) |
| S | B/(K*62.4) | calculate_volume_fly_ash_S() | Fly ash volume (ft³) |
| T | C/(L*62.4) | calculate_volume_silica_fume_T() | Silica fume volume (ft³) |
| U | D/(M*62.4) | calculate_volume_other_scm_U() | Other SCM volume (ft³) |
| V | (F/100)*27 | calculate_volume_air_V() | Air volume (ft³) |
| W | Q/62.4 | calculate_volume_water_W() | Water volume (ft³) |
| X | 27-R-S-T-U-V-W | calculate_aggregate_volume_X() | Total aggregate volume (ft³) |
| Y | 62.4*(G/100)*N*X | calculate_weight_fine_aggregate_Y() | Fine aggregate weight (lb/yd³) |
| Z | 62.4*(H/100)*O*X | calculate_weight_coarse_aggregate_Z() | Coarse aggregate weight (lb/yd³) |
| AA | 62.4*(I/100)*P*X | calculate_weight_other_aggregate_AA() | Other aggregate weight (lb/yd³) |

  - Input Parameters

The Python program will prompt users to enter project number, concrete class, and 16 numeric values (A through P to mirror corresponding Excel cell inputs). Each prompt will occur one by one using Python’s input() function, in the same order as the NDOT Excel worksheet. Table 2 lists all input parameters with their Excel cell references, units, and the Python functions that use them.

Table 2 Input Parameters

| Var. | Excel Cell | Description |
| --- | --- | --- |
| A | E10 | Weight of cement |
| B | E11 | Weight of fly ash |
| C | E12 | Weight of silica fume |
| D | E13 | Weight of other SCM |
| E | E14 | Target water-cement ratio |
| F | E15 | Target air content |
| G | E16 | Fine aggregate proportion |
| H | E17 | Coarse aggregate proportion |
| I | E18 | Other aggregate proportion |
| J | J10 | Specific gravity -cement |
| K | J11 | Specific gravity -fly ash |
| L | J12 | Specific gravity -silica fume |
| M | J13 | Specific gravity -other SCM |  |
| N | J16 | Specific gravity -fine aggregate |  |
| O | J17 | Specific gravity -coarse aggregate |  |
| P | J18 | Specific gravity -other aggregate |  |

For any materials not included in a specific mix design scenario, the user should enter “0” when prompted for a value corresponding to that material. The three aggregate proportions, fine (G), coarse (H), and other (I) must add up to 100%. Specific gravity values, J through P, must be a realistic non-zero number, even when the matching material weight is zero, because these values appear in the denominator of the volume calculations, and a zero entry would cause an error. The specific gravity values may not be found within design mix sources, so industry standard values are acceptable.

  - Output Parameters

After all inputs are entered, the program calculates 11 output values and prints a clearly formatted weight summary  chart showing the amount of each material needed to produce one cubic yard of concrete. Table 3 lists each output with its corresponding Excel cell, units, and the formula used to calculate the value.

Table 3   Output Parameters

| Var. | Excel Cell | Description | Excel Formula |
| --- | --- | --- | --- |
| Q | E20 | Weight of water | (A+B+C+D)*E |
| R | G23 | Volume of cement | A/(J*62.4) |
| S | G24 | Volume of fly ash | B/(K*62.4) |
| T | G25 | Volume of silica fume | C/(L*62.4) |
| U | G26 | Volume of other SCM | D/(M*62.4) |
| V | G27 | Volume of air | (F/100)*27 |
| W | G28 | Volume of water | Q/62.4 |
| X | G30 | Total aggregate volume | 27-R-S-T-U-V-W |
| Y | G32 | Weight of fine aggregate | 62.4*(G/100)*N*X |
| Z | G34 | Weight of coarse aggregate | 62.4*(H/100)*O*X |
| AA | G36 | Weight of other aggregate | 62.4*(I/100)*P*X |

  - Mix Design Scenarios

The following concrete mix scenarios were researched and selected to test the program. Each of the scenarios utilizes a different type of NDOT concrete mix design with its own set of input values, that are sourced from NDOT Standard Specifications for Highway Construction, Section 1002.02 (NDOT, 2017). The four scenarios used are:

- Scenario 1: NDOT Class 47B-25 - Standard pavement concrete

- Scenario 2: NDOT Class 47BD-30 - Bridge deck concrete

- Scenario 3: NDOT Class AX-25 - High-durability pavement concrete

- Scenario 4: NDOT Class PR1-25 - Rapid repair / high early strength concrete

Each scenario was run through both the Python program and the original NDOT Excel workbook using the same input values. The results were compared to verify that the Python program produces correct outputs. Each scenario produced an identical result in both Python and the original NDOT Excel workbook.

- Results and Discussion

  - Summary of Mix Design

Table 4 shows the input parameters and calculated outputs for each mix design. The results were verified against the NDOT Excel workbook and correctly matched exactly for every scenario.

Table 4 Four Mix Design Scenarios

| Parameter | Sc. 1 47B-25 | Sc. 2 47BD-30 | Sc. 3 AX-25 | Sc. 4 PR1-25 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Cementitious Materials — Inputs (lb/yd³) |  |  |  |  |  |  |
| Cement (A) | 480 | 447 | 560 | 752 |  |  |
| Fly Ash (B) | 83 | 106 | 100 | 0 |  |  |
| Silica Fume (C) | 0 | 0 | 0 | 0 |  |  |
| Other SCM (D) | 0 | 0 | 0 | 0 |  |  |
| Total Cementitious | 563 | 553 | 660 | 752 |  |  |
| Design Parameters — Inputs |  |  |  |  |  |  |
| Water-Cement Ratio (E) | 0.45 | 0.42 | 0.44 | 0.38 |  |  |
| Air Content — F (%) | 6 | 6 | 6 | 5.5 |  |  |
| Fine Aggregate — G (%) | 42 | 44 | 40 | 45 |  |  |
| Coarse Aggregate — H (%) | 58 | 56 | 60 | 55 |  |  |
| Calculated Outputs |  |  |  |  |  |  |
| Water — Q (lb/yd³) | 253.3 | 232.3 | 290.4 | 285.8 |  |  |
| Agg. Volume — X (ft³/yd³) | 18.3 | 18.6 | 16.98 | 16.25 |  |  |
| Fine Agg. — Y (lb/yd³) | 1,270.90 | 1,356.60 | 1,136.40 | 1,273.20 |  |  |
| Coarse Agg. — Z (lb/yd³) | 1,788.20 | 1,759.20 | 1,736.70 | 1,585.50 |  |  |
| Other Agg. — AA (lb/yd³) | 0 | 0 | 0 | 0 |  |  |

  - Scenario 1 - NDOT Class 47B-25 (Standard Pavement Concrete)

The first scenario, Class 47B-25 is the standard mix used for general pavement construction on Nebraska highways (NDOT, 2017). This combines 480 lb/yd³ of cement with 83 lb/yd³ of fly ash, totaling cementitious content of 563 lb/yd³. Using fly ash to replace a portion of the cement is common practice as it can reduce the cost and can improve long-term durability. The water-cement ratio of 0.45 and 6.0% air content are typical for pavement concrete in climates with freeze-thaw cycles. The aggregate blend is 42% fine and 58% coarse.

The program calculated a water demand of 253.3 lb/yd³ and a total aggregate volume of 18.30 ft³/yd³, producing fine and coarse aggregate weights of 1,270.9 and 1,788.2 lb/yd³. Scenario 1 sits in the middle of the four mixes in terms of cementitious content and aggregate volume.

  - Scenario 2 - NDOT Class 47BD-30 (Bridge Deck Concrete)

Scenario 2, Class 47BD-30 is designed for bridge decks, where reducing the permeability of the concrete is important because bridge decks are regularly exposed to salts, which can cause corrosion of the reinforcing steel below the surface (NDOT, 2017). Compared to Scenario 1, the cement content is slightly lower at 447 lb/yd³, while the fly ash content increases to 106 lb/yd³ (about 19% of the total cementitious material). The lower water-cement ratio of 0.42, reduces the amount of water in the mix which results in a denser, less permeable paste.

Scenario 2 has the lowest water weight of the four scenarios at 232.3 lb/yd³, which is a direct result of the lower water-concrete ratio. With less cementitious paste, more space is available for aggregate, giving Scenario 2 the highest aggregate volume at 18.60 ft³/yd³. This results in a fine aggregate weight of 1,356.6 lb/yd³ and a coarse aggregate weight of 1,759.2 lb/yd³, both among the highest of the four scenarios.

  - Scenario 3 - NDOT Class AX-25 (High-Durability Pavement Concrete)

The third scenario, Class AX-25 is a high-durability pavement mix with significantly more cementitious material than the other three scenarios - 560 lb/yd³ of cement plus 100 lb/yd³ of fly ash, for a total of 660 lb/yd³ (NDOT, 2017). More cementitious material means more paste, which takes up more of the available volume per cubic yard and leaves less room for aggregate. As a result, Scenario 3 has a noticeably lower aggregate volume (16.98 ft³/yd³) compared to Scenarios 1 and 2.

Even though the water-cement ratio of 0.44 is close to Scenario 1, the higher total cementitious mass drives water demand up to 290.4 lb/yd³, the highest of all four scenarios. The mix uses a coarser aggregate blend (40% fine, 60% coarse), which is typical for high-durability mixes because using less fine aggregate helps reduce drying shrinkage over time. Fine and coarse aggregate weights are 1,136.4 and 1,736.7 lb/yd³ for this mix.

  - Scenario 4 - NDOT Class PR1-25 (Rapid Repair / High Early Strength)

For the last scenario, Class PR1-25 is used for pavement patching and emergency repairs where the concrete needs to reach usable strength quickly so the road can be reopened (NDOT, 2017). To achieve high early strength, this mix uses the highest cement content of all four scenarios at 752 lb/yd³ and no fly ash. Fly ash is left out because it slows down early strength gain, which would work against the purpose of a rapid repair. The water-cement ratio of 0.38 and is the lowest of the four scenarios, which maximizes strength but also makes the mix stiffer and harder to place without admixtures.

Since there is no fly ash, the water demand calculation simplifies to Q = 752 × 0.38 = 285.8 lb/yd³. The large cement volume significantly reduces the space available for aggregate, giving Scenario 4 the lowest aggregate volume at 16.25 ft³/yd³. This results in fine and coarse aggregate weights of 1,273.2 and 1,585.5 lb/yd³ - the lowest coarse aggregate weight of the four mixes. The slightly lower air content of 5.5% is also typical for high-strength mixes, since additional air tends to reduce compressive strength.

  - Key Observations

Comparing scenarios, the two patterns are clear. First, as the total amount of cementitious material increases, the volume available for aggregate decreases. Scenario 4 has the highest cementitious content (752 lb/yd³) and the smallest aggregate volume (16.25 ft³/yd³), while Scenario 2 has the lowest cementitious content (553 lb/yd³) and the largest aggregate volume (18.60 ft³/yd³). Second, the water-cement ratio has a direct effect on water demand relative to the amount of cementitious material. Scenario 2 achieves the lowest water weight (232.3 lb/yd³) despite having a similar cementitious total to Scenario 1, simply because its w/c ratio is lower (0.42 vs. 0.45).

All four scenarios produced results through the Python program that matched the NDOT Excel workbook outputs exactly. This confirms that the formula translation was done correctly and that the program can reliably reproduce the Excel mix design calculations. The modular structure of the notebook also makes it easy to test new mix designs in the future without changing any of the core calculation functions (NDOT, 2019).

- References

- Nebraska Department of Transportation. (2017). Standard specifications for highway construction. NDOT Materials & Research Division. https://dot.nebraska.gov/media/g4qp4y0d/2017-specbook.pdf

- Nebraska Department of Transportation. (2019). Final report: Optimized aggregate gradations for Portland cement concrete mix designs evaluation (Report No. P336). NDOT Materials & Research Division. https://dot.nebraska.gov/media/3isfdv45/final-report-p336.pdf

- Nebraska Department of Transportation. (2024a). Concrete mix design submittal worksheet [Excel workbook]. NDOT Materials & Research Division. https://dot.nebraska.gov/media/jp3paote/mix-design-submittal.xlsx

- Nebraska Department of Transportation. (2024b). Project #2: Automation of concrete mix design [Course client prompt]. CIVE 202 — Civil Engineering Analysis II, University of Nebraska.

- Neville, A. M. (2011). Properties of concrete (5th ed.). Pearson Education.