| Global Constants |  |
| --- | --- |
| Python Code | Annotations |
| cubic_yard_ft3 = 27 | This line of code defines the constant cubic_yard_ft3 and assigns the integer value 27. The constant represents the conversion factor from one (1) cubic yard to cubic feet. |
| unit_weight_water = 62.4 | This line of code defines the constant unit_weight_water and assigns a floating-point value of 62.4. |
| print(f"Defined Constant: 1 cubic yard = {cubic_yard_ft3} cubic ft") | This line of code prints the defined constant for cubic_yard_ft3 as an f-string to display the assigned value of 27 cubic yards. |
| print(f"Defined Constant: Unit weight of water = {unit_weight_water} lb/ft³") | This line of code prints the defined constant for unit_weight_water as an f-string to display the assigned value of 62.4 lb/ft3. |

| Function Definitions |  |
| --- | --- |
| Python Code | Annotations |
| def calculate_water_weight_Q(weight_cement_A, weight_fly_ash_B, weight_silica_fume_C, weight_other_scm_D, ratio_water_cement_E): water_weight_Q = (weight_cement_A + weight_fly_ash_B + weight_silica_fume_C + weight_other_scm_D) * ratio_water_cement_E return water_weight_Q | This line of code uses def to define the function calculate_water_weight_Q. The function accepts parameters within the parentheses that correspond to Excel cells A, B, C, D, and E from the original NDOT Excel worksheet. The four cementitious materials (weight_cement_A, weight_fly_ash_B, weight_silica_fume_C, weight_other_scm_D) are then summed inside the next set of parentheses and that sum is multiplied by the water-cement ratio (ratio_water_cement_E) to calculate the weight of water (water_weight_Q). The return statement sends the calculated value back to the location where the function was called from and the value is now available for future calculations. |
| def calculate_volume_cement_R(weight_cement_A, sg_cement_J): volume_cement_R = weight_cement_A / (sg_cement_J * unit_weight_water) return volume_cement_R | This line of code uses def to define the function calculate_volume_cement_R. The function accepts parameters within the parentheses that correspond to Excel cells A and J from the original NDOT Excel worksheet. The weight of cement (weight_cement_A) is divided by the product of the unit weight of water (unit_weight_water) and specific gravity of cement (sg_cement_J) to calculate the volume of cement (volume_cement_R). The return statement sends the calculated value back to the location where the function was called from and the value is now available for future calculations. |
| def calculate_volume_fly_ash_S(weight_fly_ash_B, sg_fly_ash_K): volume_fly_ash_S = weight_fly_ash_B / (sg_fly_ash_K * unit_weight_water) return volume_fly_ash_S | This line of code uses def to define the function calculate_volume_fly_ash_S. The function accepts parameters within the parentheses that correspond to Excel cells B and K from the original NDOT Excel worksheet. The weight of fly ash (weight_fly_ash_B) is divided by the product of the specific gravity of fly ash (sg_fly_ash_K) and the unit weight of water (unit_weight_water) to calculate the volume of fly ash (volume_fly_ash_S). The return statement sends the calculated value back to the location where the function was called from and the value is now available for future calculations. |
| def calculate_volume_silica_fume_T(weight_silica_fume_C, sg_silica_fume_L): volume_silica_fume_T = weight_silica_fume_C / (sg_silica_fume_L * unit_weight_water) return volume_silica_fume_T | This line of code uses def to define the function calculate_volume_silica_fume_T. The function accepts parameters within the parentheses that correspond to Excel cells C and L from the original NDOT Excel worksheet. The weight of silica fume (weight_silica_fume_C) is divided by the product of the specific gravity of silica fume (sg_silica_fume_L) and the unit weight of water (unit_weight_water) to calculate the volume of silica fume (volume_silica_fume_T). The return statement sends the calculated value back to the location where the function was called from and the value is now available for future calculations. |
| def calculate_volume_other_scm_U(weight_other_scm_D, sg_other_scm_M): volume_other_scm_U = weight_other_scm_D / (sg_other_scm_M * unit_weight_water) return volume_other_scm_U | This line of code uses def to define the function calculate_volume_other_scm_U. The function accepts parameters within the parentheses that correspond to Excel cells D and M from the original NDOT Excel worksheet. The weight of other SCM (weight_other_scm_D) is divided by the product of the specific gravity of other SCM (sg_other_scm_M) and the unit weight of water (unit_weight_water) to calculate the volume of other SCM (volume_other_scm_U). The return statement sends the calculated value back to the location where the function was called from and the value is now available for future calculations. |
| def calculate_volume_air_V(air_content_F): volume_air_V = (air_content_F / 100) * cubic_yard_ft3 return volume_air_V | This line of code uses def to define the function calculate_volume_air_V. The function accepts a parameter within the parentheses that corresponds to Excel cell F from the original NDOT Excel worksheet. The air content percentage (air_content_F) is divided by 100 to convert it to a decimal, then multiplied by the defined constant cubic_yard_ft3 to calculate the volume of air (volume_air_V). The return statement sends the calculated value back to the location where the function was called from and the value is now available for future calculations. |
| def calculate_volume_water_W(water_weight_Q): volume_water_W = water_weight_Q / unit_weight_water return volume_water_W | This line of code uses def to define the function calculate_volume_water_W. The function accepts a parameter within the parentheses that corresponds to Excel cell Q from the original NDOT Excel worksheet. The water weight (water_weight_Q) is divided by the constant for the weight of water (unit_weight_water) to calculate the volume of water (volume_water_W). The return statement sends the calculated value back to the location where the function was called from and the value is now available for future calculations. |
| def calculate_aggregate_volume_X(volume_cement_R, volume_fly_ash_S, volume_silica_fume_T, volume_other_scm_U, volume_air_V, volume_water_W): aggregate_volume_X = (cubic_yard_ft3 - volume_cement_R - volume_fly_ash_S - volume_silica_fume_T - volume_other_scm_U - volume_air_V - volume_water_W) return aggregate_volume_X | This line of code uses def to define the function calculate_aggregate_volume_X. The function accepts parameters within the parentheses that corresponds to Excel cells R, S, T, U, V, and W from the original NDOT Excel worksheet. Starting with the constant cubic_yard_ft3, the volumes of all non-aggregate components (volume_cement_R, volume_fly_ash_S, volume_silica_fume_T, volume_other_scm_U, volume_air_V, volume_water_W) are subtracted sequentially to calculate the total volume available for aggregate (aggregate_volume_X). The return statement sends the calculated value back to the location where the function was called from and the value is now available for future calculations. |
| def calculate_weight_fine_aggregate_Y(percent_fine_G, sg_fine_N, aggregate_volume_X): weight_fine_aggregate_Y = (unit_weight_water * (percent_fine_G / 100) * sg_fine_N * aggregate_volume_X) return weight_fine_aggregate_Y | This line of code uses def to define the function calculate_weight_fine_aggregate_Y. The function accepts parameters within the parentheses that corresponds to Excel cells G, N, and X from the original NDOT Excel worksheet. The fine aggregate proportion (percent_fine_G) is divided by 100 to convert from a percentage to a decimal, then multiplied by the unit weight of water (unit_weight_water), the specific gravity of fine aggregate (sg_fine_N), and the total aggregate volume (aggregate_volume_X) to calculate the weight of fine aggregate (weight_fine_aggregate_Y). The return statement sends the calculated value back to the location where the function was called from and the value is now available for future calculations. |
| def calculate_weight_coarse_aggregate_Z(percent_coarse_H, sg_coarse_O, aggregate_volume_X): weight_coarse_aggregate_Z = (unit_weight_water * (percent_coarse_H / 100) * sg_coarse_O * aggregate_volume_X) return weight_coarse_aggregate_Z | This line of code uses def to define the function calculate_weight_coarse_aggregate_Z. The function accepts parameters within the parentheses that corresponds to Excel cells H, O, and X from the original NDOT Excel worksheet. The coarse aggregate proportion (percent_coarse_H) is divided by 100 to convert from a percentage to a decimal, then multiplied by the unit weight of water (unit_weight_water), the specific gravity of coarse aggregate (sg_coarse_O), and the total aggregate volume (aggregate_volume_X) to calculate the weight of fine aggregate (weight_coarse_aggregate_Z). The return statement sends the calculated value back to the location where the function was called from and the value is now available for future calculations. |
| def calculate_weight_other_aggregate_AA(percent_other_I, sg_other_P, aggregate_volume_X): weight_other_aggregate_AA = (unit_weight_water * (percent_other_I / 100) * sg_other_P * aggregate_volume_X) return weight_other_aggregate_AA | This line of code uses def to define the function calculate_weight_other_aggregate_AA. The function accepts parameters within the parentheses that corresponds to Excel cells I, P, and X from the original NDOT Excel worksheet. The other aggregate proportion (percent_other_I) is divided by 100 to convert from a percentage to a decimal, then multiplied by the unit weight of water (unit_weight_water), the specific gravity of other aggregate (sg_other_P), and the total aggregate volume (aggregate_volume_X) to calculate the weight of other aggregate (weight_other_aggregate_AA). The return statement sends the calculated value back to the location where the function was called from and the value is now available for future calculations. |

| User Input |  |
| --- | --- |
| Python Code | Annotations |
| project_no = int(input("Enter Project Number: ")) | This line of code uses the input() function to prompt the user to “Enter Project Number: “. The int() function converts the text entered into an integer. The result is stored in project_no to be used for the output weight summary chart. |
| concrete_class = input("Enter Class of Concrete: ") | This line of code uses the input() function to prompt the user to “Enter Class of Concrete: “. The result is stored in concrete_class to be used for the output weight summary chart. |
| cement_A = float(input("Enter cement weight A (lb/yd³): ")) | This line of code uses the input() function to prompt the user to “Enter cement weight A (lb/ yd³): “. The float() function converts the text entered into a decimal number. The result is stored in cement_A to be used in calculations. |
| fly_ash_B = float(input("Enter fly ash weight B (lb/yd³): ")) | This line of code uses the input() function to prompt the user to “Enter fly ash weight B (lb/ yd³): “. The float() function converts the text entered into a decimal number. The result is stored in fly_ash_B to be used in calculations. |
| silica_fume_C = float(input("Enter silica fume weight C (lb/yd³): ")) | This line of code uses the input() function to prompt the user to “Enter silica fume weight C (lb/ yd³): “. The float() function converts the text entered into a decimal number. The result is stored in silica_fume_C to be used in calculations. |
| other_scm_D = float(input("Enter other SCM weight D (lb/yd³): ")) | This line of code uses the input() function to prompt the user to “Enter other SCM weight D (lb/ yd³): “. The float() function converts the text entered into a decimal number. The result is stored in other_scm_D to be used in calculations. |
| water_cement_ratio_E = float(input("Enter target water-cement ratio E: ")) | This line of code uses the input() function to prompt the user to “Enter target water-cement ratio E: “. The float() function converts the text entered into a decimal number. The result is stored in water_cement_ratio_E to be used in calculations. |
| air_content_F = float(input("Enter target air content F (%): ")) | This line of code uses the input() function to prompt the user to “Enter target air content F (%): “. The float() function converts the text entered into a decimal number. The result is stored in air_content_F to be used in calculations. |
| percent_fine_G = float(input("Enter target percent fine aggregate G (%): ")) | This line of code uses the input() function to prompt the user to “Enter target percent fine aggregate G (%): “. The float() function converts the text entered into a decimal number. The result is stored in percent_fine_G to be used in calculations. |
| percent_coarse_H = float(input("Enter target percent coarse aggregate H (%): ")) | This line of code uses the input() function to prompt the user to “Enter target percent coarse aggregate H (%): “. The float() function converts the text entered into a decimal number. The result is stored in percent_coarse_H to be used in calculations. |
| percent_other_I = float(input("Enter target percent other aggregate I (%): ")) | This line of code uses the input() function to prompt the user to “Enter target percent other aggregate I (%): “. The float() function converts the text entered into a decimal number. The result is stored in percent_other_I to be used in calculations. |
| sg_cement_J = float(input("Enter specific gravity of cement J: ")) | This line of code uses the input() function to prompt the user to “Enter specific gravity of cement J: “. The float() function converts the text entered into a decimal number. The result is stored in sg_cement_J to be used in calculations. |
| sg_fly_ash_K = float(input("Enter specific gravity of fly ash K: ")) | This line of code uses the input() function to prompt the user to “Enter specific gravity of fly ash K: “. The float() function converts the text entered into a decimal number. The result is stored in sg_fly_ash_K to be used in calculations. |
| sg_silica_fume_L = float(input("Enter specific gravity of silica fume L: ")) | This line of code uses the input() function to prompt the user to “Enter specific gravity of silica fume L: “. The float() function converts the text entered into a decimal number. The result is stored in sg_silica_fume_L to be used in calculations. |
| sg_other_scm_M = float(input("Enter specific gravity of other SCM M: ")) | This line of code uses the input() function to prompt the user to “Enter specific gravity of other SCM M: “. The float() function converts the text entered into a decimal number. The result is stored in sg_other_scm_M to be used in calculations. |
| sg_fine_N = float(input("Enter specific gravity of fine aggregate N: ")) | This line of code uses the input() function to prompt the user to “Enter specific gravity of fine aggregate N: “. The float() function converts the text entered into a decimal number. The result is stored in sg_fine_N to be used in calculations. |
| sg_coarse_O = float(input("Enter specific gravity of coarse aggregate O: ")) | This line of code uses the input() function to prompt the user to “Enter specific gravity of coarse aggregate O: “. The float() function converts the text entered into a decimal number. The result is stored in sg_coarse_O to be used in calculations. |
| sg_other_P = float(input("Enter specific gravity of other aggregate P: ")) | This line of code uses the input() function to prompt the user to “Enter specific gravity of other aggregate P: “. The float() function converts the text entered into a decimal number. The result is stored in sg_other_P to be used in calculations. |

| Sequential Calculations |  |
| --- | --- |
| Python Code | Annotations |
| Q = calculate_water_weight_Q(cement_A, fly_ash_B, silica_fume_C, other_scm_D, water_cement_ratio_E) | This line of code calls the function calculate_water_weight_Q() and uses the variables (cement_A, fly_ash_B, silica_fume_C, other_scm_D, water_cement_ratio_E) in the calculation. This returns the value of Water Weight and is stored in the variable Q. |
| print(f"Water Weight (Q): {Q:.1f} lb/yd³") | This line of code prints the stored value of the variable Q as an f-string and the format specifier :.1f rounds the output to one decimal place. |
| R = calculate_volume_cement_R(cement_A, sg_cement_J) | This line of code calls the function calculate_volume_cement_R() and uses the variables (cement_A and sg_cement_J) in the calculation. This returns the value for Cement Volume and is stored in the variable R. |
| S = calculate_volume_fly_ash_S(fly_ash_B, sg_fly_ash_K) | This line of code calls the function calculate_volume_fly_ash_S() and uses the variables (fly_ash_B and sg_fly_ash_K) in the calculation. This returns the value for Fly Ash Volume and is stored in the variable S. |
| T = calculate_volume_silica_fume_T(silica_fume_C, sg_silica_fume_L) | This line of code calls the function calculate_volume_silica_fume_T() and uses the variables (silica_fume_C and sg_silica_fume_L) in the calculation. This returns the value for Silica Fume Volume and is stored in the variable T. |
| U = calculate_volume_other_scm_U(other_scm_D, sg_other_scm_M) | This line of code calls the function calculate_volume_other_scm_U() and uses the variables (other_scm_D and sg_other_scm_M) in the calculation. This returns the value for Other SCM Volume and is stored in the variable U. |
| print(f"Cement volume (R): {R:.3f} ft³") | This line of code prints the stored value of the variable R as an f-string and the format specifier :.3f rounds the output to three decimal places. |
| print(f"Fly ash volume (S): {S:.3f} ft³") | This line of code prints the stored value of the variable S as an f-string and the format specifier :.3f rounds the output to three decimal places. |
| print(f"Silica fume volume (T): {T:.3f} ft³") | This line of code prints the stored value of the variable T as an f-string and the format specifier :.3f rounds the output to three decimal places. |
| print(f"Other SCM volume (U): {U:.3f} ft³") | This line of code prints the stored value of the variable U as an f-string and the format specifier :.3f rounds the output to three decimal places. |
| V = calculate_volume_air_V(air_content_F) | This line of code calls the function calculate_volume_air_V() and uses the variable (air_content_F) in the calculation. This returns the value for Air Volume and is stored in the variable R. |
| W = calculate_volume_water_W(Q) | This line of code calls the function calculate_volume_water_W() and uses the variable (Q) in the calculation. This returns the value for Water Volume and is stored in the variable W. |
| print(f"Air volume (V): {V:.3f} ft³") | This line of code prints the stored value of the variable V as an f-string and the format specifier :.3f rounds the output to three decimal places. |
| print(f"Water volume (W): {W:.3f} ft³") | This line of code prints the stored value of the variable W as an f-string and the format specifier :.3f rounds the output to three decimal places. |
| X = calculate_aggregate_volume_X(R, S, T, U, V, W) | This line of code calls the function calculate_aggregate_volume_X() and uses the variables (R, S, T, U, V, W) in the calculation. This returns the value for Available Aggregate Volume and is stored in the variable X. |
| print(f"Total aggregate volume (X): {X:.3f} ft³") | This line of code prints the stored value of the variable X as an f-string and the format specifier :.3f rounds the output to three decimal places. |
| Y = calculate_weight_fine_aggregate_Y(percent_fine_G, sg_fine_N, X) | This line of code calls the function calculate_weight_fine_aggregate_Y() and uses the variables (percent_fine_G, sg_fine_N, and X) in the calculation. This returns the value for Fine Aggregate Weight and is stored in the variable Y. |
| Z = calculate_weight_coarse_aggregate_Z(percent_coarse_H, sg_coarse_O, X) | This line of code calls the function calculate_weight_coarse_aggregate_Z() and uses the variables (percent_coarse_H, sg_coarse_O, and X) in the calculation. This returns the value for Coarse Aggregate Weight and is stored in the variable Y. |
| AA = calculate_weight_other_aggregate_AA(percent_other_I, sg_other_P, X) | This line of code calls the function calculate_weight_other_aggregate_AA() and uses the variables (percent_other_I, sg_other_P, and X) in the calculation. This returns the value for Other Aggregate Weight and is stored in the variable AA. |
| print(f"Fine aggregate weight (Y): {Y:.1f} lb/yd³") | This line of code prints the stored value of the variable Y as an f-string and the format specifier :.1f rounds the output to one decimal places. |
| print(f"Coarse aggregate weight (Z): {Z:.1f} lb/yd³") | This line of code prints the stored value of the variable Z as an f-string and the format specifier :.1f rounds the output to one decimal places. |
| print(f"Other aggregate weight (AA): {AA:.1f} lb/yd³") | This line of code prints the stored value of the variable Z as an f-string and the format specifier :.1f rounds the output to one decimal places. |

| Mix Design Weight Summary |  |
| --- | --- |
| Python Code | Annotations |
| print("\n================================================") print(" NDOT Concrete Mix Design — Weight Summary") print(" (Per 1 Cubic Yard of Concrete)") print("================================================") | These print statements create a header block for the weight summary. The first line uses \n to insert a blank line for separation. The equal signs create a border to separate the header block from the rest of the weight summary. The two lines of text create the title and subtitle to identify the report and the unit of measure. |
| print(f" Project Number : {project_no}") print(f" Concrete Class : {concrete_class}") | These two lines of code print the user inputed project number and concrete class as f-strings. |
| print("------------------------------------------------") | This line of code prints a dashed line to separate sections of the weight summary. |
| print(f" Cement (A) : {cement_A:8.1f} lb/yd³") | This line of code prints the calculated cement weight as an f-string with a format specifier which makes the total width 8 characters and rounds to 1 decimal place. |
| print(f" Fly Ash (B) : {fly_ash_B:8.1f} lb/yd³") | This line of code prints the calculated fly ash weight as an f-string with a format specifier which makes the total width 8 characters and rounds to 1 decimal place. |
| print(f" Silica Fume (C): {silica_fume_C:8.1f} lb/yd³") | This line of code prints the calculated silica fume weight as an f-string with a format specifier which makes the total width 8 characters and rounds to 1 decimal place. |
| print(f" Other SCM (D) : {other_scm_D:8.1f} lb/yd³") | This line of code prints the calculated other SCM weight as an f-string with a format specifier which makes the total width 8 characters and rounds to 1 decimal place. |
| print("------------------------------------------------") | This line of code prints a dashed line to separate sections of the weight summary. |
| print(f" Fine (Y) : {Y:8.1f} lb/yd³") | This line of code prints the calculated fine aggregate weight as an f-string with a format specifier which makes the total width 8 characters and rounds to 1 decimal place. |
| print(f" Coarse (Z) : {Z:8.1f} lb/yd³") | This line of code prints the calculated coarse aggregate weight as an f-string with a format specifier which makes the total width 8 characters and rounds to 1 decimal place. |
| print(f" Other (AA) : {AA:8.1f} lb/yd³") | This line of code prints the calculated other aggregate weight as an f-string with a format specifier which makes the total width 8 characters and rounds to 1 decimal place. |
| print("------------------------------------------------") | This line of code prints a dashed line to separate sections of the weight summary. |
| print(f" Water (Q) : {Q:8.1f} lb/yd³") | This line of code prints the calculated water weight as an f-string with a format specifier which makes the total width 8 characters and rounds to 1 decimal place. |
| print("================================================") print(" All weights per 1 cubic yard of concrete.") print("================================================") | These print statements create a footer block for the weight summary. The equal signs create a border to separate the footer block from the rest of the weight summary. |

| Concrete Mix Design Scenario Testing – Scenario 1 |  |
| --- | --- |
| Python Code | Annotations |
| project_no = 1 | This line of code assigns the project number as a hardcoded value this replaces the value from the previously used input() prompts. |
| concrete_class = "NDOT Class 47B-25 (Standard Pavement Concrete)" | This line of code assigns the concrete class as a hardcoded value this replaces the value from the previously used input() prompts. |
| cement_A = 480 fly_ash_B = 83 silica_fume_C = 0 other_scm_D = 0 | These lines of code assign each cementitious material a weight for scenario 1. This replaces the values from the previous user input section. |
| water_cement_ratio_E = 0.45 air_content_F = 6.0 | These lines of code set the design paramaters for Scenario 1. These replace the values from the previous user input section. |
| percent_fine_G = 42 percent_coarse_H = 58 percent_other_I = 0 | These lines of code assign the aggregate proportions (%) for scenario 1. This replaces the values from the previous user input section. |
| sg_cement_J = 3.15 sg_fly_ash_K = 2.30 sg_silica_fume_L = 2.20 sg_other_scm_M = 2.50 sg_fine_N = 2.65 sg_coarse_O = 2.70 sg_other_P = 2.65 | These lines of code assign values for specific gravity of all materials. These are industry standard values and replace the values from the previous user input section. |
| Q = calculate_water_weight_Q(cement_A, fly_ash_B, silica_fume_C, other_scm_D, water_cement_ratio_E) R = calculate_volume_cement_R(cement_A, sg_cement_J) S = calculate_volume_fly_ash_S(fly_ash_B, sg_fly_ash_K) T = calculate_volume_silica_fume_T(silica_fume_C, sg_silica_fume_L) U = calculate_volume_other_scm_U(other_scm_D, sg_other_scm_M) V = calculate_volume_air_V(air_content_F) W = calculate_volume_water_W(Q) X = calculate_aggregate_volume_X(R, S, T, U, V, W) Y = calculate_weight_fine_aggregate_Y(percent_fine_G, sg_fine_N, X) Z = calculate_weight_coarse_aggregate_Z(percent_coarse_H, sg_coarse_O, X) AA = calculate_weight_other_aggregate_AA(percent_other_I, sg_other_P, X) | These lines of code call each of the 11 calculation functions in the required dependency order. These calculation functions will now use the hardcoded Scenario 1 values as arguments. |
| print("\n================================================") # ... (weight summary print block) | The lines of code for the weight summary print block are identical to the previous weight summary. See “Mix Design Weight Summary” section above for details. |

| Concrete Mix Design Scenario Testing – Scenario 2 |  |
| --- | --- |
| Python Code | Annotations |
| project_no = 2 | This line of code assigns the project number as a hardcoded value this replaces the value from scenario 1. |
| concrete_class = "NDOT Class 47BD-30 (Bridge Deck Concrete)” | This line of code assigns the concrete class as a hardcoded value this replaces the value from scenario 1. |
| cement_A = 447 fly_ash_B = 106 silica_fume_C = 0 other_scm_D = 0 | These lines of code assign each cementitious material a weight for scenario 2. This replaces the values from scenario 1. |
| water_cement_ratio_E = 0.42 air_content_F = 6.0 | These lines of code set the design paramaters for scenario 2. These replace the values from scenario 1. |
| percent_fine_G = 44 percent_coarse_H = 56 percent_other_I = 0 | These lines of code assign the aggregate proportions (%) for scenario 2. This replaces the values from scenario 1. |
| sg_cement_J = 3.15 sg_fly_ash_K = 2.30 sg_silica_fume_L = 2.20 sg_other_scm_M = 2.50 sg_fine_N = 2.65 sg_coarse_O = 2.70 sg_other_P = 2.65 | These lines of code assign values for specific gravity of all materials. These are industry standard values and replace the values from the previous user input section. |
| Q = calculate_water_weight_Q(cement_A, fly_ash_B, silica_fume_C, other_scm_D, water_cement_ratio_E) R = calculate_volume_cement_R(cement_A, sg_cement_J) S = calculate_volume_fly_ash_S(fly_ash_B, sg_fly_ash_K) T = calculate_volume_silica_fume_T(silica_fume_C, sg_silica_fume_L) U = calculate_volume_other_scm_U(other_scm_D, sg_other_scm_M) V = calculate_volume_air_V(air_content_F) W = calculate_volume_water_W(Q) X = calculate_aggregate_volume_X(R, S, T, U, V, W) Y = calculate_weight_fine_aggregate_Y(percent_fine_G, sg_fine_N, X) Z = calculate_weight_coarse_aggregate_Z(percent_coarse_H, sg_coarse_O, X) AA = calculate_weight_other_aggregate_AA(percent_other_I, sg_other_P, X) | These lines of code call each of the 11 calculation functions in the required dependency order. These calculation functions will now use the hardcoded Scenario 2 values as arguments. |
| print("\n================================================") # ... (weight summary print block) | The lines of code for the weight summary print block are identical to the previous weight summary. See “Mix Design Weight Summary” section above for details. |

| Concrete Mix Design Scenario Testing – Scenario 3 |  |
| --- | --- |
| Python Code | Annotations |
| project_no = 3 | This line of code assigns the project number as a hardcoded value this replaces the value from scenario 2. |
| concrete_class = "NDOT Class AX-25 (High-Durability Pavement Concrete)” | This line of code assigns the concrete class as a hardcoded value this replaces the value from scenario 2. |
| cement_A = 560 fly_ash_B = 100 silica_fume_C = 0 other_scm_D = 0 | These lines of code assign each cementitious material a weight for scenario 3. This replaces the values from scenario 2. |
| water_cement_ratio_E = 0.44 air_content_F = 6.0 | These lines of code set the design paramaters for scenario 3. These replace the values from scenario 2. |
| percent_fine_G = 40 percent_coarse_H = 60 percent_other_I = 0 | These lines of code assign the aggregate proportions (%) for scenario 3. This replaces the values from scenario 2. |
| sg_cement_J = 3.15 sg_fly_ash_K = 2.30 sg_silica_fume_L = 2.20 sg_other_scm_M = 2.50 sg_fine_N = 2.65 sg_coarse_O = 2.70 sg_other_P = 2.65 | These lines of code assign values for specific gravity of all materials. These are industry standard values and replace the values from the previous user input section. |
| Q = calculate_water_weight_Q(cement_A, fly_ash_B, silica_fume_C, other_scm_D, water_cement_ratio_E) R = calculate_volume_cement_R(cement_A, sg_cement_J) S = calculate_volume_fly_ash_S(fly_ash_B, sg_fly_ash_K) T = calculate_volume_silica_fume_T(silica_fume_C, sg_silica_fume_L) U = calculate_volume_other_scm_U(other_scm_D, sg_other_scm_M) V = calculate_volume_air_V(air_content_F) W = calculate_volume_water_W(Q) X = calculate_aggregate_volume_X(R, S, T, U, V, W) Y = calculate_weight_fine_aggregate_Y(percent_fine_G, sg_fine_N, X) Z = calculate_weight_coarse_aggregate_Z(percent_coarse_H, sg_coarse_O, X) AA = calculate_weight_other_aggregate_AA(percent_other_I, sg_other_P, X) | These lines of code call each of the 11 calculation functions in the required dependency order. These calculation functions will now use the hardcoded Scenario 3 values as arguments. |
| print("\n================================================") # ... (weight summary print block) | The lines of code for the weight summary print block are identical to the previous weight summary. See “Mix Design Weight Summary” section above for details. |

| Concrete Mix Design Scenario Testing – Scenario 4 |  |
| --- | --- |
| Python Code | Annotations |
| project_no = 4 | This line of code assigns the project number as a hardcoded value this replaces the value from scenario 3. |
| concrete_class = "NDOT Class 47BD-30 (Bridge Deck Concrete)” | This line of code assigns the concrete class as a hardcoded value this replaces the value from scenario 3. |
| cement_A = 752 fly_ash_B = 0 silica_fume_C = 0 other_scm_D = 0 | These lines of code assign each cementitious material a weight for scenario 4. This replaces the values from scenario 3. |
| water_cement_ratio_E = 0.38 air_content_F = 5.5 | These lines of code set the design paramaters for scenario 4. These replace the values from scenario 3. |
| percent_fine_G = 45 percent_coarse_H = 55 percent_other_I = 0 | These lines of code assign the aggregate proportions (%) for scenario 4. This replaces the values from scenario 3. |
| sg_cement_J = 3.15 sg_fly_ash_K = 2.30 sg_silica_fume_L = 2.20 sg_other_scm_M = 2.50 sg_fine_N = 2.65 sg_coarse_O = 2.70 sg_other_P = 2.65 | These lines of code assign values for specific gravity of all materials. These are industry standard values and replace the values from the previous user input section. |
| Q = calculate_water_weight_Q(cement_A, fly_ash_B, silica_fume_C, other_scm_D, water_cement_ratio_E) R = calculate_volume_cement_R(cement_A, sg_cement_J) S = calculate_volume_fly_ash_S(fly_ash_B, sg_fly_ash_K) T = calculate_volume_silica_fume_T(silica_fume_C, sg_silica_fume_L) U = calculate_volume_other_scm_U(other_scm_D, sg_other_scm_M) V = calculate_volume_air_V(air_content_F) W = calculate_volume_water_W(Q) X = calculate_aggregate_volume_X(R, S, T, U, V, W) Y = calculate_weight_fine_aggregate_Y(percent_fine_G, sg_fine_N, X) Z = calculate_weight_coarse_aggregate_Z(percent_coarse_H, sg_coarse_O, X) AA = calculate_weight_other_aggregate_AA(percent_other_I, sg_other_P, X) | These lines of code call each of the 11 calculation functions in the required dependency order. These calculation functions will now use the hardcoded Scenario 4 values as arguments. |
| print("\n================================================") # ... (weight summary print block) | The lines of code for the weight summary print block are identical to the previous weight summary. See “Mix Design Weight Summary” section above for details. |